# ESP32_HandRobot_Inmoov — Key Features Deep Dive

# 1. Real-Time Hand Tracking → Servo Control Pipeline

What it does: Your laptop webcam captures your hand, MediaPipe detects 21 3D landmarks, the Python code computes how curled each finger is (0–100%), maps that to servo angles (0–180°), and sends them to the ESP32 which drives the physical robot hand — all in a continuous real-time loop.

The full signal chain:

Webcam (1280×720)
  → cv2.VideoCapture reads BGR frame
  → Flip if mirror mode
  → Convert BGR → RGB
  → MediaPipe Hand Landmarker (Tasks API, VIDEO mode)
  → 21 normalized (x,y,z) landmarks
  → Per-finger curl calculation (0–100%)
  → EMA smoothing + anti-shake filtering
  → curl_to_servo_angle() with grip limiting
  → Deadband filter (skip if <3° change)
  → Rate-limited send (30 Hz max)
  → Serial/WiFi → ESP32
  → ESP32 smooth interpolation (8°/tick max)
  → PCA9685 I2C PWM driver
  → 6× MG996R servo motors
  → Fishing-line tendons pull 3D-printed fingers

Key code locations:

Frame capture & main loop: main.py lines 735–783

MediaPipe detection: hand_tracker.py process_frame() lines 150–228

Curl → angle mapping: main.py curl_to_servo_angle() lines 153–184

How finger curl is computed:

For regular fingers (index, middle, ring, pinky): measures the angle at the PIP and DIP joints using the dot-product formula. A weighted sum (65% PIP + 35% DIP) maps the combined joint angle to a 0–100% curl value. 170° = fully straight (0%), 40° = fully bent (100%).

For thumb: uses a hybrid of distance + angle because the thumb's opposition motion doesn't follow the same flex pattern. It measures (a) the normalized distance from thumb tip to index MCP, and (b) the MCP joint angle, then averages both methods 50/50.

Note: All math is done with pure Python scalar arithmetic (no numpy arrays) to avoid per-frame allocation overhead — important for real-time performance.

# 2. Dual Communication — Serial + WiFi

What it does: The Python client can talk to the ESP32 over either USB Serial (wired, reliable, ~0 latency) or WiFi TCP socket (wireless, ~2-5ms latency). Both are active simultaneously on the ESP32, so you can monitor Serial output while controlling via WiFi.

Architecture:

Python Client                          ESP32 Firmware
┌──────────────┐                     ┌──────────────────┐
│ BaseClient   │ ◄── abstract ──►    │                  │
│  ├ SerialClient ──── USB ────────► │ handleSerial()   │
│  └ WiFiClient   ──── TCP ────────► │ handleWiFi()     │
│                  │                  │                  │
│ create_client()  │  Factory         │ processCommand() │
│ (reads config)   │                  │ (shared parser)  │
└──────────────┘                     └──────────────────┘

| Aspect | Serial | WiFi |
| --- | --- | --- |
| Transport | USB UART (CH340 chip) | TCP socket |
| Config | SERIAL_PORT = "COM6", BAUD = 115200 | ESP32_IP = "192.168.1.61", PORT = 8080 |
| Latency | ~1 ms | ~2–5 ms (LAN) |
| Setup | Plug USB cable | Flash WiFi credentials, same network |
| Python class | SerialClient (esp32_client.py) | WiFiClient (esp32_client.py) |

The protocol is a compact text format with single-letter command prefixes:

| Command | Format | Purpose |
| --- | --- | --- |
| F | F<t>,<i>,<m>,<r>,<p>,<w>\n | Set all 6 servos |
| C | C<ch>,<angle>\n | Set single servo (calibration) |
| P | P\n | Ping → responds PONG |
| S | S\n | Status query → angles + grip |
| G | G<strength>\n | Set grip strength (0–100%) |
| I | I<inv0>,…,<inv5>\n | Sync inversion flags |
| M | M<min0>,…,<min5>\n | Sync min angles |
| X | X<max0>,…,<max5>\n | Sync max angles |

Rate limiting & deadband: The BaseClient.send_all_servos() method skips sending if (a) less than 33ms has passed since the last send, or (b) no servo angle changed by more than DEADBAND_DEGREES (2°). This prevents flooding the ESP32 with redundant commands.

# 3. Multi-Layer Anti-Shake / Stabilization

What it does: Raw MediaPipe landmark data is inherently noisy — landmarks jitter by a few pixels between frames even when your hand is perfectly still. Without filtering, this causes servos to buzz and vibrate constantly. The system applies four layers of stabilization to eliminate this.

## Layer 1: Velocity-Adaptive EMA Smoothing (Python)

Standard EMA applies a fixed smoothing factor (alpha = 0.35). But this project uses velocity-adaptive smoothing — it detects whether a finger is actually moving or just jittering:

If the curl change is > 4% (intentional movement): use alpha = 0.35 → responsive tracking

If the curl change is ≤ 4% (micro-jitter): use alpha = 0.12 → much heavier smoothing, kills noise

# From hand_tracker.py process_frame()
delta = abs(raw[key] - self._smoothed[key])
alpha = alpha_fast if delta > threshold else alpha_slow  # 0.35 vs 0.12
self._smoothed[key] = alpha * raw[key] + (1 - alpha) * self._smoothed[key]

## Layer 2: Snap-to-Rest (Python)

When a finger's smoothed curl drops below 2.5%, it's snapped to exactly 0.0%. This prevents tiny oscillations around the "fully open" position from ever reaching the servos.

for finger in ("thumb", "index", "middle", "ring", "pinky"):
    if self._smoothed[finger] < snap_rest:
        self._smoothed[finger] = 0.0

## Layer 3: Per-Servo Output Deadband (Python)

After computing the servo angle, apply_servo_deadband() compares it against the last actually sent angle. If the difference is less than 3°, the new angle is rejected and the old stable angle is kept. This catches residual jitter that passed through EMA.

if old_val is None or abs(new_val - old_val) >= deadband:
    stable_angles[ch] = new_val   # Accept
    result[ch] = new_val
else:
    result[ch] = old_val           # Reject — hold stable

## Layer 4: Firmware-Side Deadband (ESP32)

Even after all Python filtering, the firmware has its own deadband (SERVO_DEADBAND_DEG = 2). If the target angle differs from the current position by less than 2°, the servo update is skipped entirely — no PWM signal change occurs.

#if SERVO_DEADBAND_DEG > 0
    if (abs(effectiveTarget - currentAngle[i]) < SERVO_DEADBAND_DEG) {
      continue;  // Skip this servo entirely
    }
#endif

Summary of filter chain:

Raw MediaPipe landmarks (noisy, ±2–5° jitter)
  → Layer 1: Velocity-adaptive EMA (kills fast jitter, preserves movement)
  → Layer 2: Snap-to-rest (eliminates idle oscillation near 0%)
  → Layer 3: Python servo deadband (3° threshold)
  → Layer 4: Firmware deadband (2° threshold)
  → Clean, stable servo output (no buzzing)

# 4. Grip Protection System

What it does: Prevents the robot hand from crushing objects by limiting how far fingers can close, and progressively slowing them down as they approach the limit. This is critical because MG996R servos produce ~10 kg·cm of torque — enough to crush a glass.

## Grip Modes

Four presets, cyclable with G key or by clicking the grip card:

| Mode | Strength | Compliance Zone | Use Case |
| --- | --- | --- | --- |
| DELICATE | 35% | 40° | Eggs, glass, fragile items |
| LIGHT | 55% | 30° | Plastic cups, soft objects |
| NORMAL | 75% | 20° | General use |
| FIRM | 100% | 10° | No limit, full grip |

Strength caps the maximum curl percentage a finger can reach. At 35%, fingers can only close to 35% of their full range — they physically cannot squeeze harder.

Fine-tuning: Press +/- to adjust strength in 5% increments within any mode.

Per-finger overrides: In config.py, you can set individual finger limits (e.g., thumb at 100% while other fingers follow the mode).

## Compliance Zone (Soft Landing)

The compliance zone is the deceleration ramp before the grip limit. Instead of fingers suddenly stopping at the limit (which can cause a force spike), they progressively slow down using a cosine easing curve:

# From main.py curl_to_servo_angle()
zone_progress = (curl_pct - (max_curl - compliance)) / compliance
easing = 0.5 * (1.0 - math.cos(math.pi * zone_progress))  # Cosine ease-in
effective_curl = (max_curl - compliance) + compliance * easing

This creates a smooth, gradual approach — like a car braking smoothly rather than slamming into a wall.

## Firmware-Side Compliance

The ESP32 firmware has its own compliance zone (COMPLIANCE_ZONE_DEG = 20). When a servo is closing and within 20° of its target, the interpolation speed is linearly reduced to a minimum of 1°/update:

int scaledSpeed = COMPLIANCE_MIN_SPEED +
  (absDiff * (SERVO_SPEED_LIMIT - COMPLIANCE_MIN_SPEED)) / COMPLIANCE_ZONE_DEG;

## Optional Stall/Current Detection

If you add hardware current sensing (shunt resistor + ADC on GPIO 34), the firmware can detect when a servo is stalling (drawing excessive current, meaning it hit an obstacle) and automatically back off by 15°:

if (adcVal > CURRENT_STALL_ADC) {
    stallCounter++;
    if (stallCounter >= STALL_DEBOUNCE_COUNT) {
        // Back off all closing servos
        targetAngle[i] = currentAngle[i] - STALL_BACKOFF_DEG;
        stallDetected[i] = true;
    }
}

# 5. Hold Mode (Posture Lock)

What it does: When you press H (or click the Hold badge) while a hand is detected, the system freezes the current finger positions. The servos maintain that exact posture indefinitely — even if you remove your hand from the camera. Press H again to release back to live tracking.

How it works internally:

1. When activating hold, three things are captured and stored:

held_curls — the current curl percentages for all fingers

held_servo_angles — the servo angles being sent

held_landmarks — the 21 MediaPipe landmarks (for the pose thumbnail)

2. While hold is active, the main loop ignores live hand tracking and instead uses the frozen values:

if state['hold_mode'] and state['held_servo_angles'] is not None:
    curls = dict(state['held_curls'])            # Use frozen
    servo_angles = dict(state['held_servo_angles'])  # Use frozen
else:
    # Normal live tracking...

3. The UI shows a yellow "POSE LOCKED" card with the frozen hand skeleton, so you can visually confirm what posture is being held.

Safety: Hold mode requires a hand to be currently detected. If no hand is visible, pressing H prints a warning and does nothing — this prevents accidentally locking a "no hand" state.

# 6. Interactive HUD Overlay

What it does: The camera feed is overlaid with a full heads-up display built using OpenCV drawing primitives — semi-transparent cards, clickable badges, telemetry bars, and status alerts. Everything is mouse-interactive.

## UI Components

Top Badge Bar:

QUIT — red-bordered badge, clickable to exit

ACTIVE / PAUSED — green/red toggle, clickable

INMOOV — title badge with blue border

CONNECTED / DISCONNECTED — clickable to trigger reconnect

FPS — live frame rate (green if ≥15, orange if low)

Hold Badge (below top bar): Shows "POSTURE LOCKED" (yellow) or "LIVE CONTROLLER" (gray). Clickable to toggle hold mode.

Grip Controller Card (top-right): Shows current grip mode label and strength percentage. Visual progress bar with color coding (green/orange/red). Clickable to cycle grip modes. Corner accent decorations for sci-fi styling.

Center Alerts:

"NO HAND DETECTED" warning (orange border) when hand is lost

"SYSTEM TRANSMISSION: PAUSED" alert (red border) when paused — clickable to resume

Telemetry Card (bottom-left): 5 (or 6 with wrist) vertical bar gauges showing real-time curl/angle for each finger. Color-coded: green (0–35%), orange (35–70%), red (70–100%). Wrist toggle button inside the card. Each bar shows the servo angle value above it and a letter label below.

Pose Thumbnail Card (bottom-right): Normalized 2D hand skeleton rendered inside a bounded card. Shows live hand pose or the frozen hold pose. Joints colored by type (red=wrist, blue=tips, green=joints).

Bottom Taskbar: Dark strip showing all keyboard shortcuts as a quick reference.

## Mouse Click System

All interactive elements register their bounding boxes in a global CLICKABLE_REGIONS dict. A single on_mouse_click callback checks which region was clicked:

for region_name, (rx1, ry1, rx2, ry2) in CLICKABLE_REGIONS.items():
    if rx1 <= x <= rx2 and ry1 <= y <= ry2:
        # Handle click for this region

Semi-transparent cards use alpha blending on a small ROI copy (not the full frame) for performance.

# 7. Wrist Rotation Control

What it does: Detects the roll (pronation/supination) of your palm and maps it to a 6th servo controlling the robot's wrist rotation. Toggleable with Space key.

The math (detailed):

This is the most mathematically complex part of the project. The challenge is isolating the forearm roll from arm tilt or wrist flexion.

Step 1: Define the hand's longitudinal axis — a unit vector from wrist (landmark 0) to middle finger MCP (landmark 9).

Step 2: Compute the cross-palm vector — from index MCP (landmark 5) to pinky MCP (landmark 17). This vector rotates as you pronate/supinate your wrist.

Step 3: Project the cross-palm vector onto the plane perpendicular to the longitudinal axis, removing the component along the hand axis:

c_perp = c - (c · a) * a

Step 4: Build two orthogonal reference axes in the perpendicular plane using Gram-Schmidt orthogonalization against a world-frame reference (camera-right (1,0,0) or camera-up (0,-1,0) as fallback).

Step 5: Decompose the projected cross-palm vector into components along these two axes and compute the angle using atan2:

roll_rad = math.atan2(comp_u, comp_r)

Step 6: Map roll to servo using cosine:

servo_angle = (math.cos(roll_rad) + 1.0) * 90.0  # 0–180°

This gives: palm facing camera → 180°, back of hand → 0°, with smooth transitions.

# 8. Servo Calibration Tool

What it does: An interactive GUI tool for finding each servo's mechanical limits (min/max angles) and direction. Essential because each servo + finger mechanism has different physical constraints.

The tool provides:

6 trackbar sliders — one per servo, draggable from 0° to 180°

Visual angle bars with green (MIN) and blue (MAX) markers

Selected servo highlighting — number keys 1–6 to select

Real-time servo control — moving a slider immediately moves the physical servo

Calibration workflow:

| Key | Action |
| --- | --- |
| 1–6 | Select a servo |
| Drag slider | Move the servo to find positions |
| O | Save current position as MIN (open) |
| C | Save current position as MAX (closed) |
| I | Toggle inversion (if servo moves backwards) |
| T | Test selected servo — sweeps MIN → MAX → MIN |
| A | Test all — wave pattern across all fingers |
| S | Save to calibration_data.json + print config values |

Test functions run in background threads so the UI stays responsive while servos are sweeping.

# 9. Config Sync Protocol (Python → ESP32)

What it does: At startup, the Python client pushes its calibration settings to the ESP32 firmware so both sides enforce the same safety bounds. This means you only need to edit config.py — the firmware automatically receives the values.

What gets synced (in order):

| Step | Command | Data Sent | Purpose |
| --- | --- | --- | --- |
| 1 | I | Inversion flags (0/1 × 6) | Which servos run backwards |
| 2 | M | Min angles (× 6) | Open position limits |
| 3 | X | Max angles (× 6) | Closed position limits |
| 4 | G | Grip strength (0–100) | Current grip mode limit |

After syncing, the firmware recalculates gripLimitAngle[] for every servo based on the new min/max/inversion + current grip strength.

def sync_config(self):
    # Send Inversions
    invs = [1 if config.SERVO_INVERTED[f] else 0 for f in fingers]
    self._send_raw(f"I{invs[0]},{invs[1]},{invs[2]},{invs[3]},{invs[4]},{invs[5]}\n")
    # Send Min angles
    mins = [config.SERVO_MIN[f] for f in fingers]
    self._send_raw(f"M{mins[0]},{mins[1]},{mins[2]},{mins[3]},{mins[4]},{mins[5]}\n")
    # Send Max angles
    maxs = [config.SERVO_MAX[f] for f in fingers]
    self._send_raw(f"X{maxs[0]},{maxs[1]},{maxs[2]},{maxs[3]},{maxs[4]},{maxs[5]}\n")

# 10. Firmware-Side Safety & Smooth Interpolation

What it does: The ESP32 firmware doesn't just blindly set servo positions. It independently enforces safety limits, smoothly interpolates movement, and has its own compliance system — even if the Python client crashes or sends bad data.

## Smooth Servo Interpolation

Instead of jumping directly to target angles (which causes mechanical stress and jerky motion), the firmware moves servos at most 8° per update cycle (every 20ms):

#define SERVO_SPEED_LIMIT  8    // Max degrees per update
#define UPDATE_INTERVAL_MS 20   // Update every 20ms = 50 Hz

if (abs(diff) > speedLimit) {
    diff = (diff > 0) ? speedLimit : -speedLimit;
}
currentAngle[i] += diff;

At 50 Hz with 8°/tick, a full 180° sweep takes ~450ms — smooth enough to look natural but fast enough to feel responsive.

## Failsafe Angle Clamping

Every servo update is clamped to the synced min/max range, regardless of what command was received:

int low = safetyMin[i] < safetyMax[i] ? safetyMin[i] : safetyMax[i];
int high = safetyMin[i] > safetyMax[i] ? safetyMin[i] : safetyMax[i];
if (currentAngle[i] < low) currentAngle[i] = low;
else if (currentAngle[i] > high) currentAngle[i] = high;

## Grip Limit Enforcement

The firmware maintains its own gripLimitAngle[] array. During updateServos(), every target angle is capped to the grip limit before any movement happens — so even if the Python client ignores grip protection, the firmware won't let the servo exceed the limit.

## Status LED Feedback

The built-in ESP32 LED (GPIO 2) indicates system state:

Fast blink (200ms) — No WiFi connection

Slow blink (1000ms) — WiFi connected, waiting for client

Solid on — Client actively connected

## I2C Optimization

The I2C bus runs at 400kHz Fast Mode (vs default 100kHz) for faster PCA9685 register writes, enabling quicker PWM updates to all 6 servos.

# Hardware Architecture

┌─────────────────────────────────────────────────────────────┐
│                    Laptop (Python)                           │
│  ┌──────────┐  ┌──────────┐  ┌───────────┐  ┌──────────┐  │
│  │ Webcam   │→ │MediaPipe │→ │ main.py   │→ │esp32_    │  │
│  │ 1280×720 │  │ Tasks    │  │ curl→servo│  │client.py │  │
│  └──────────┘  └──────────┘  └───────────┘  └─────┬────┘  │
│                                                     │       │
└─────────────────────────────────────────────────────┼───────┘
                              USB Serial (115200) OR  │ WiFi TCP (:8080)
┌─────────────────────────────────────────────────────┼───────┐
│                    ESP32 DevKit 30P                  │       │
│  ┌────────────────────────┐                         │       │
│  │ esp32_hand_controller  │ ◄───────────────────────┘       │
│  │  • Command parser      │                                  │
│  │  • Smooth interpolation│                                  │
│  │  • Grip limiting       │                                  │
│  │  • Safety clamping     │                                  │
│  └──────────┬─────────────┘                                  │
│        I2C (SDA=21, SCL=22, 400kHz)                          │
└─────────────┼────────────────────────────────────────────────┘
┌─────────────┼────────────────────────────────────────────────┐
│        PCA9685 16-Ch PWM Driver (0x40)                       │
│  ┌────┐ ┌────┐ ┌────┐ ┌────┐ ┌────┐ ┌────┐                 │
│  │CH0 │ │CH1 │ │CH2 │ │CH3 │ │CH4 │ │CH5 │                 │
│  │Thum│ │Indx│ │Mid │ │Ring│ │Pnky│ │Wrst│  ← MG996R ×6    │
│  └────┘ └────┘ └────┘ └────┘ └────┘ └────┘                 │
│                    ↑ V+ from 5V 20A PSU                      │
└──────────────────────────────────────────────────────────────┘